# Template 4EB00 Q1 2026

# Special Topic: The Jet Engine

|  | Naam | Studentnr |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

?

Groep

| Name | Value | Unit |
| --- | --- | --- |
| Fuel |  |  |
| Tamb |  | [K] |
| P3overP2 |  | [-] |
| Pamb |  | [Pa] |
| mfurate |  | [kg/s] |
| AF |  | [kg/kg] |
| v1 |  | [m/s] |

Assignment

Conditions

(Give your settings)

## Assignment Cycle Analysis of the Jet Engine based on NASA tables

Applying conservation equations and the NASA tables fill out table 1. The code and approach should be explained in detail in the following pages. As an example a worked-out part of the procedure for the diffusor (page 5) is given. You should still add the code snippet and provide the actual line numbers.

| State | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| P (kPa) |  |  |  | 502.89 |  |  |
| T (K) |  |  |  | 1002.81 |  |  |
| v (m/s) |  |  |  | 0 |  |  |
| Table 1: Thermodynamic state summary |  |  |  |  |  |  |

## Diffusor

You do not have to fill this box. It will not be graded.

Code snippet diffusor [1-2] the one in the sheets. Don’t need yours here. Work starts at compressor stage

### Procedure (This just an Example. Your work starts at the compressor stage.)

Solved is first (line nr)

to determine T2 the isentropic relation can be used to solve for the pressure yielding

.

Which is implemented in (line nr). Mixture properties determined using f.i. (line nr)

The composition Yi is taken equal to that of normal air.

## Compressor

Code snippet Compressor [2-3]

### Procedure

## Combustor (2 code snippets)

147 %% [3-4] Combustor :: composition before and after the combustor 148 % Needs T3 and P3 from the compressor. Fuel and air both enter at T3. 149 sPart = 'Combustor'; 150 mairrate = AF*mfurate; % Air mass flow, AF = mair/mfuel [kg/s] 151 mtotrate = mairrate+mfurate; % Mass conservation: flow leaving [kg/s] 152 Yreac = (AF*Yair+Yfuel)/(AF+1); % Unburnt mixture: AF kg air + 1 kg fuel 153 % Complete combustion: CxHy + nuO2 O2 -> x CO2 + y/2 H2O 154 ElF = SpS(1).Elcomp; % Atoms in the fuel, order [O H C N Ar] 155 nC = ElF(3);nH = ElF(2); % x and y: number of C and H atoms 156 nuO2 = nC+nH/4; % Moles O2 needed per mole of fuel 157 AFstoi = nuO2*Mi(2)/Mi(1)/Yair(2); % Stoichiometric air-fuel ratio [kg/kg] 158 phi = AFstoi/AF; % Equivalence ratio (<1: excess air) 159 nfuel = Yreac(1)/Mi(1); % Moles of fuel per kg unburnt mixture 160 Yprod = Yreac; % Burnt mixture, N2 does not react 161 Yprod(1) = 0; % All fuel is burnt 162 Yprod(2) = Yreac(2)-nuO2*nfuel*Mi(2); % O2 that is left over 163 Yprod(3) = Yreac(3)+nC*nfuel*Mi(3); % CO2 that is formed 164 Yprod(4) = Yreac(4)+nH/2*nfuel*Mi(4); % H2O that is formed 165 Rgreac = Runiv*sum(Yreac./Mi); % Gas constant unburnt mixture [J/kg/K] 166 Rgprod = Runiv*sum(Yprod./Mi); % Gas constant burnt mixture [J/kg/K]

Code snippet computation of composition before and after combustor [3-4]

167 %% [3-4] Combustor :: thermodynamic computations 168 P4 = P3; % Combustion at constant pressure 169 v3 = 0;v4 = 0; % Velocities in the engine are neglected 170 for i=1:NSp 171 hi3(i) = HNasa(T3,SpS(i)); % hi(T3), formation enthalpy included 172 end 173 hreac = Yreac*hi3'; % Enthalpy of the unburnt mixture at T3 174 h4 = hreac+0.5*v3^2-0.5*v4^2; % Energy balance: adiabatic, no work 175 hprod_a = Yprod*hia'; % Enthalpy burnt mixture for range of T 176 T4 = interp1(hprod_a,TR,h4); % Interpolate h4 on hprod_a to get T4 177 for i=1:NSp 178 hi4(i) = HNasa(T4,SpS(i)); 179 end 180 h4check = Yprod*hi4'; % Check: must give h4 back

Code snippet thermodynamic computations [3-4]

### Procedure

The combustor is treated as a steady flow control volume with two inlets (the air from the compressor at state 3 and the fuel) and one outlet (state 4). It is adiabatic, no work is done, the pressure is constant and the velocities are neglected (v3 = v4 = 0, like v2). The fuel enters at T3 and burns completely. Mass conservation gives the mass flows (lines 150-151)

mair=AF⋅mfuel,  m4=mair+mfuel

so 1 kg of fuel is mixed with AF kg of air. The mass fractions before combustion are then (line 152)

Yi,reac=AF⋅Yi,air+Yi,fuelAF+1

The fuel reacts with the oxygen of the air. The numbers of C and H atoms in the fuel, x and y, are read from the database (lines 154-155), for gasoline this gives x = 7.76 and y = 13.1. The reaction is

CxHy+x+y4 O2→x CO2+y2 H2O

The oxygen needed for 1 kg of fuel gives the stoichiometric air-fuel ratio and the equivalence ratio (lines 156-158)

AFst=x+y4⋅MO2Mfuel⋅YO2,air,  φ=AFstAF

Because φ < 1 there is excess air, so all the fuel is consumed and O2 is left over. One kg of mixture contains nfuel = Yfuel,reac / Mfuel moles of fuel (line 159). For every mole of fuel (x + y/4) moles of O2 disappear and x moles of CO2 and y/2 moles of H2O are formed, which gives the mass fractions after combustion (lines 160-164)

YO2,prod=YO2,reac−x+y4⋅nfuel⋅MO2

YCO2,prod=x⋅nfuel⋅MCO2,  YH2O,prod=y2⋅nfuel⋅MH2O

The fuel mass fraction becomes zero and N2 does not react, so its mass fraction stays the same. The specific gas constants of both mixtures follow from the mass fractions (lines 165-166)

Rg=RuiYiMi

Energy conservation for the control volume, divided by the total mass flow, gives (lines 170-174)

iYi,prodhiT4=iYi,reachiT3+12v32−12v42

The left hand side is the enthalpy of the burnt mixture at T4 and the sum on the right is the enthalpy of the unburnt mixture at T3. The enthalpies from HNasa include the enthalpy of formation, so the heat that is released by the reaction is already part of this balance and no heating value is needed. The products have a lower enthalpy of formation than the reactants and this difference shows up as a temperature rise. The enthalpy of the burnt mixture is computed for the whole range TR (line 175) and T4 is found by interpolating h4 on this curve (line 176). As a check the enthalpy of the products is computed again at T4 (lines 177-180), which returns h4. The pressure follows from the constant pressure assumption, P4 = P3 (line 168). The isentropic relation is not used in this stage, because combustion is irreversible.

Result: T4 = 1002.81 K and P4 = 502.89 kPa. The composition before and after the combustor and the gas constants are given in table 2.

Complete table 2 (don’t forget to assign values to the red question marks)

|  | AF=71.25 (equivalence ratio =0.200 ) |  |
| --- | --- | --- |
| Mass fractions | Initial | Final |
| Fuel | 0.01384 | 0 |
| O2 | 0.22969 | 0.18376 |
| N2 | 0.75647 | 0.75647 |
| CO2 | 0 | 0.04443 |
| H2O | 0 | 0.01535 |
| Rg [J/kg/K] | 285.29 | 287.75 |
| Table 2: Mixture composition before and after the combustor and the specific gas constants. |  |  |

## Turbine

Code snippet Turbine [4-5]

### Procedure

## Nozzle

Code snippet Nozzle [5-6]

### Procedure